# Section 5: Code Walkthrough — Simulating Fourier Optics in Python

Everything we have discussed so far — mask as a signal, diffraction as Fourier transform, lens as a filter, image reconstruction as inverse Fourier transform — can be simulated in under 100 lines of pure Python.

The full notebook is available on GitHub: github.com/nishantbhadauria/photolithography-fourier-optics. It runs on Google Colab with no installs — pure numpy and matplotlib.

We walk through each block below, explaining what the code does and why.

## Setup — No Special Installs Needed

The simulation uses only standard scientific Python libraries:

import numpy as np

import matplotlib.pyplot as plt

import matplotlib.gridspec as gridspec

## Block 1: The Mask as a Signal

We model the mask as a 1D binary grating — a repeating pattern of ones (quartz, light passes) and zeros (chrome, light blocked). This is the simplest mask pattern — the building block of all lithography.

N = 1024                    # number of spatial samples

x = np.linspace(0, 1, N)   # normalised spatial coordinate

line_pairs = 10             # 10 line-space pairs across mask

# Binary grating

mask = 0.5 * (1 + np.sign(np.sin(2 * np.pi * line_pairs * x)))

The key insight: this is just a square wave in space. The same mathematical object you would see in a digital signal processing course — except here it represents physical chrome and quartz on glass.

Mask pattern output:

1 |  #   #   #   #   #   #   #   #   #   #

|  #   #   #   #   #   #   #   #   #   #

0 |## ### ### ### ### ### ### ### ### ### ##

+-----------------------------------------> position

chrome  quartz  chrome  quartz ...

## Block 2: Diffraction = Fourier Transform

We take the FFT of the mask. This is the mathematical equivalent of what light does physically when it diffracts through the mask openings — decomposing the spatial pattern into frequency components.

# FFT of mask — equivalent to physical diffraction

mask_fft = np.fft.fftshift(np.fft.fft(mask))

freqs    = np.fft.fftshift(np.fft.fftfreq(N))

magnitude = np.abs(mask_fft)

fftshift centres the spectrum so zero frequency is in the middle — matching the physical picture where the 0th order goes straight through and higher orders fan outward symmetrically.

The output is a series of spikes — each spike is a diffraction order. For a grating of 10 line pairs, you see spikes at 0 (0th order), ±10, ±20, ±30... in normalised frequency units. Each carries information about the mask pattern.

Diffraction orders in frequency domain:

Magnitude

|

300|        *                   *

|

100|    *       *           *       *

|  *   *   *   *   *   *   *   *   *

0 +-----------------------------------------> frequency

-0.15  -0.10  -0.05   0   0.05  0.10  0.15

-2nd  -1st   0th  +1st  +2nd

## Block 3: Lens as a Low-Pass Filter

The lens aperture sits at the Fourier plane and physically blocks diffraction orders beyond the NA cutoff. We model this as a simple frequency domain filter — a rectangular window that zeroes out everything beyond the cutoff.

def apply_na_filter(fft_signal, freqs, na):

cutoff = na * 0.5   # NA=1.0 passes half the spectrum

pupil  = (np.abs(freqs) <= cutoff).astype(float)

return fft_signal * pupil, pupil

We apply this filter at three NA values — 0.85, 0.55, and 0.25 — to visualise how the available frequency content shrinks as NA decreases. The red regions in the plot are information lost forever — no amount of post-processing can recover them.

NA = 0.85  -> passes 0th, ±1st, ±2nd, ±3rd, ±4th orders

NA = 0.55  -> passes 0th, ±1st, ±2nd orders only

NA = 0.25  -> passes 0th, ±1st order only

NA = 0.05  -> passes 0th order only -> flat image, no pattern

## Block 4: Image Reconstruction — Inverse FFT

The lens recombines the surviving frequency components onto the wafer. Mathematically this is the inverse FFT of the filtered spectrum.

# Inverse FFT of filtered spectrum -> reconstructed image

filtered_fft, _ = apply_na_filter(mask_fft, freqs, na)

reconstructed   = np.real(np.fft.ifft(np.fft.ifftshift(filtered_fft)))

# Normalise to [0, 1]

reconstructed = (reconstructed - reconstructed.min())

reconstructed = reconstructed / reconstructed.max()

The side-by-side comparison of all three NA cases makes the resolution degradation viscerally clear:

High NA (0.85):   |_| |_| |_|   sharp edges, high contrast

Medium NA (0.55): / \ / \ / \   rounded edges, medium contrast

Low NA (0.25):    ~  ~  ~  ~    barely visible, low contrast

Original mask:    |_| |_| |_|   reference

## Block 5: The Resolution Limit — Rayleigh Criterion from Simulation

Finally we sweep NA continuously from 0.05 to 0.99 and measure Michelson contrast — the ratio of bright-to-dark variation in the reconstructed image:

# Michelson contrast = (Imax - Imin) / (Imax + Imin)

contrast = (recon.max() - recon.min()) / (recon.max() + recon.min())

The resulting contrast vs NA curve shows the resolution limit emerging naturally — no formula assumed, no Rayleigh criterion imposed. The physics produces it automatically. Below a critical NA, contrast drops below the process threshold (0.2) and the pattern becomes unresolvable.

This is the Rayleigh criterion — not as a formula memorised, but as an observable consequence of the Fourier optics pipeline. The simulation shows why the limit exists, not just what it is.

## The Full Pipeline in One View

Step         Physics              Math             Code

----------------------------------------------------------------

Mask         Chrome/quartz        Binary signal    np.sign(sin)

Diffraction  Light fans out       FFT              np.fft.fft()

Lens         Aperture blocks      Low-pass filter  abs(freqs)<=cutoff

Image        Waves recombine      Inverse FFT      np.fft.ifft()

Resolution   Contrast collapses   Rayleigh limit   Michelson contrast

The entire simulation is under 150 lines. Yet it captures the essential physics of a multi-billion dollar manufacturing process. This is the power of Fourier optics — it reduces an enormously complex physical system to a sequence of linear operations that any data scientist can implement and reason about.

The notebook on GitHub extends this further — annotated plots, order-by-order analysis, and a contrast-vs-NA sweep. In Section 6, we step back and ask: why does any of this matter to a data scientist, and where does machine learning enter this picture?

Next: Section 6 — So What? Where ML Meets Lithography