# Section 2: The Physics of Light

Light exhibits both wave and particle properties. In photolithography, we utilize its wave behaviour — specifically wavelength, phase, and interference — to understand how patterns are transferred onto silicon.

## Light as a Wave

As a wave, light has three properties that matter in lithography:

- Wavelength (λ) — the distance between two successive peaks. Determines colour and, critically, resolution. Shorter λ = finer features printable.

- Amplitude — the height of the wave. Determines intensity of light — how bright.

- Phase — where in the cycle the wave currently is. This is the secret ingredient behind interference and everything interesting in optics.

### Wave Properties

Amplitude

|

/\   |   /\       /\

/  \  |  /  \     /  \

/    \ | /    \   /    \

--------+--------\/-------->  position

|

|<--λ-->|            wavelength

In lithography, the wavelength is the most critical parameter. Current systems use 193nm (ArF excimer laser) for conventional fabs and 13.5nm (EUV — extreme ultraviolet) for leading edge nodes.

## Diffraction — What Happens at an Edge

When light passes through a narrow opening or past an edge, it does not travel in a perfectly straight line — it spreads out. This is diffraction. It is not a defect or an imperfection — it is a fundamental property of waves.

The narrower the opening, the more the light spreads. This is the fundamental tension in lithography — you want sharp edges, but physics forces the light to blur them.

### Diffraction Through a Mask Opening

Light source

| | | | | | |

| | | | | | |

__|             |__

|                  |   <- mask (chrome blocks light)

\ | /

\|/             <- light spreads after gap

|

wafer below

Notice the diffracted light fans out at multiple angles — these are called diffraction orders. The 0th order goes straight through. The 1st, 2nd orders fan outward at increasing angles. Each order carries information about the mask pattern.

### Diffraction Orders

MASK          LENS PLANE          WAFER

(Pupil / Fourier

Plane)

|||||   ->    . 0th order   ->    image

. +1st order

. -1st order

. +2nd order

. -2nd order

x 3rd order (blocked by NA)

x 4th order (blocked by NA)

## Interference — When Two Waves Meet

When two waves occupy the same space, they add together. Depending on their relative phase, they either reinforce or cancel each other.

### Constructive vs Destructive Interference

In phase (constructive):     Out of phase (destructive):

/\      /\                     /\

/  \    /  \          +     \  /

/    \  /    \                \/

\/

= larger wave                = zero (cancellation)

- Constructive interference — wave peaks align → bright fringe on wafer

- Destructive interference — peak meets trough → dark fringe on wafer

The pattern on the wafer is not formed by simple projection — it is formed by the interference of diffracted waves recombining after passing through the lens. This is why the mathematics of waves, specifically Fourier analysis, governs everything in lithography.

## Fringe Spacing Formula

From Young's double slit experiment — the classic demonstration of wave interference — we get the fringe spacing formula:

Δy = λL / d

- Δy — fringe spacing — distance between bright bands on the wafer

- λ — wavelength of light

- L — distance from mask to wafer

- d — distance between the two slit openings

The key insight: smaller λ → smaller Δy → finer fringes → finer features printable. This is why the semiconductor industry relentlessly pushed toward shorter wavelengths over decades.

## Rayleigh Resolution Formula

The Rayleigh criterion gives us the minimum feature size a lens system can resolve:

R = 0.61 × λ / NA

- R — minimum resolvable feature size

- λ — wavelength of light

- NA — numerical aperture — how wide an angle of light the lens captures

Two levers to improve resolution: shrink λ or increase NA. Both have hard physical limits.

### Wavelength Progression in the Industry

Mercury lamp  436nm  (g-line)         <- 1980s

Mercury lamp  365nm  (i-line)         <- early 1990s

KrF laser     248nm                   <- mid 1990s

ArF laser     193nm                   <- 2000s, still used today

EUV           13.5nm                  <- 2019 onwards, leading edge

193nm has been the workhorse for over 20 years. The industry squeezed it further using immersion lithography (replacing air with water between lens and wafer, effectively reducing λ to ~134nm) and multiple patterning. EUV at 13.5nm was a 30-year engineering effort.

We now have the two key formulas. But there is a deeper mathematical framework connecting diffraction, interference, and image formation — one that data scientists will recognise immediately. In Section 3, we will see how the Fourier transform is not just a mathematical tool, but something light itself performs physically.

Next: Section 3 — Fourier Optics: Where the Math Meets the Light